HOW TO REACH: 941 8817 B/8817 B 1978/

941 8817 B/8817 B 1978/

DISK ON ROD/B/

N40-49-16.4 W124-10-48.5

GO SW'LY ON NEW NAVY BASE ROAD FROM RTE 255, TAKE FIRST LEFT (S'LY) ON COOKHOUSE RD. TURN LEFT (E'LY) AT VANCE RD. GO EAST 0.1 MILE TO GATE ON RIGHT. WITH ESCORT, PROCEED THRU GATE SSE 30 FEET APPROXMITLY THEN ROAD TURNS NNE PROCEED 170 FEET APRROXMITLY TO 4 FOOT BY 4 FOOT CONC PAD AND OLD STREET LIGHT ON NORTH SIDE OF EDGE OF ROAD, DISK WILL BE 8.5 FEET NNE FROM OLD STREET LIGHT AND 7 FEET NORTH FROM EDGE OF ROAD. DISK ON ROD ENCASED IN PVC DISK 0.5 FEET BELOW SURFACE.